EG114728 HD in AI and Robotics

MBS4544 Robot Sensing and Vision

Assignment 3

This assignment is submitted individually.

Deadline: 4 Dec 2023

Stage 1

You are required to build a robot car with four mecanum wheels and develop a ESP32 code to drive the car by the PS4 controller.

PS4 Controller

Demo Video:

https://vtcmca-my.sharepoint.com/:v:/g/personal/220171174_stu_vtc_edu_hk/EaBYLGn6mQFCn2PaWTtbZdsBVV-NuRBwEEURH0cs9krmjQ?e=dbbuPP

Code:

#include <PS4Controller.h>

#include "esp_bt_main.h"

#include "esp_bt_device.h"

#include "esp_gap_bt_api.h"

#include "esp_err.h"

#define FORWARD 1

#define BACKWARD 2

#define LEFT 3

#define RIGHT 4

#define FORWARD_LEFT 5

#define FORWARD_RIGHT 6

#define BACKWARD_LEFT 7

#define BACKWARD_RIGHT 8

#define ROTATE_LEFT 9

#define ROTATE_RIGHT 10

#define STOP 0

#define BACK_RIGHT_MOTOR 1

#define BACK_LEFT_MOTOR 2

#define FRONT_RIGHT_MOTOR 3

#define FRONT_LEFT_MOTOR 4

#define MAX_MOTOR_SPEED 120 // about 50% duty cycle

unsigned long lastTimeStamp = 0;

//FRONT RIGHT MOTOR

int enableFrontRightMotor = 14;

int FrontRightMotorPin1 = 26;

int FrontRightMotorPin2 = 27;

//BACK RIGHT MOTOR

int enableBackRightMotor=22;

int BackRightMotorPin1=16;

int BackRightMotorPin2=17;

//FRONT LEFT MOTOR

int enableFrontLeftMotor = 32;

int FrontLeftMotorPin1 = 33;

int FrontLeftMotorPin2 = 25;

//BACK LEFT MOTOR

int enableBackLeftMotor = 23;

int BackLeftMotorPin1 = 18;

int BackLeftMotorPin2 = 19;

const int PWMFreq = 1000; /* 1 KHz */

const int PWMResolution = 8; // 8-bit

const int PWMSpeedChannel_1 = 1; // 0-15

const int PWMSpeedChannel_2 = 2; // 0-15

const int PWMSpeedChannel_3 = 3; // 0-15

const int PWMSpeedChannel_4 = 4; // 0-15

void notify()

{

int LStickY = map( PS4.LStickY(), -128, 127, -254, 254);

int LStickX = map( PS4.LStickX(), -128, 127, -254, 254);

int RStickX = map( PS4.RStickX(), -128, 127, -254, 254);

if (LStickY > 30 && LStickX < -30) {moveCar(FORWARD_LEFT);}

else if (LStickY > 30 && LStickX > 30) {moveCar(FORWARD_RIGHT);}

else if (LStickY < -30 && LStickX < -30) {moveCar(BACKWARD_LEFT);}

else if (LStickY < -30 && LStickX > 30) {moveCar(BACKWARD_RIGHT);}

else if (LStickY > 90) {moveCar(FORWARD);}

else if (LStickY < -90) {moveCar(BACKWARD);}

else if (LStickX < -90) {moveCar(LEFT);}

else if (LStickX > 90) {moveCar(RIGHT);}

else if (RStickX < -50) {moveCar(ROTATE_LEFT);}

else if (RStickX > 50) {moveCar(ROTATE_RIGHT);}

else {moveCar(STOP);}

// Print data for debugging purpose only

if (millis() - lastTimeStamp > 100)

{

Serial.print(LStickX);

Serial.print(',');

Serial.print(LStickY);

Serial.print(',');

Serial.print(RStickX);

Serial.println();

lastTimeStamp = millis();

}

}

void onConnect()

{

Serial.println("Connected!");

}

void onDisConnect()

{

rotateMotor(BACK_RIGHT_MOTOR, 0);

rotateMotor(BACK_LEFT_MOTOR, 0);

rotateMotor(FRONT_RIGHT_MOTOR, 0);

rotateMotor(FRONT_LEFT_MOTOR, 0);

}

void moveCar(int inputValue)

{

switch(inputValue)

{

case FORWARD:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case BACKWARD:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case FORWARD_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

// rotateMotor(BACK_RIGHT_MOTOR, STOP);

// rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case FORWARD_RIGHT:

// rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

// rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

case BACKWARD_LEFT:

// rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

// rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

case BACKWARD_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

// rotateMotor(BACK_RIGHT_MOTOR, STOP);

// rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case ROTATE_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case ROTATE_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case STOP:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

default:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

}

}

void rotateMotor(int motorNumber, int motorSpeed)

{

if (motorSpeed < 0 && motorNumber == 1)

{

digitalWrite(BackRightMotorPin1,LOW);

digitalWrite(BackRightMotorPin2,HIGH);

}

else if (motorSpeed < 0 && motorNumber == 2)

{

digitalWrite(BackLeftMotorPin1,LOW);

digitalWrite(BackLeftMotorPin2,HIGH);

}

else if (motorSpeed < 0 && motorNumber == 3)

{

digitalWrite(FrontRightMotorPin1,LOW);

digitalWrite(FrontRightMotorPin2,HIGH);

}

else if (motorSpeed < 0 && motorNumber == 4)

{

digitalWrite(FrontLeftMotorPin1,LOW);

digitalWrite(FrontLeftMotorPin2,HIGH);

}

else if (motorSpeed > 0 && motorNumber == 1)

{

digitalWrite(BackRightMotorPin1,HIGH);

digitalWrite(BackRightMotorPin2,LOW);

}

else if (motorSpeed > 0 && motorNumber == 2)

{

digitalWrite(BackLeftMotorPin1,HIGH);

digitalWrite(BackLeftMotorPin2,LOW);

}

else if (motorSpeed > 0 && motorNumber == 3)

{

digitalWrite(FrontRightMotorPin1,HIGH);

digitalWrite(FrontRightMotorPin2,LOW);

}

else if (motorSpeed > 0 && motorNumber == 4)

{

digitalWrite(FrontLeftMotorPin1,HIGH);

digitalWrite(FrontLeftMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 1)

{

digitalWrite(FrontRightMotorPin1,LOW);

digitalWrite(FrontRightMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 2)

{

digitalWrite(FrontLeftMotorPin1,LOW);

digitalWrite(FrontLeftMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 3)

{

digitalWrite(BackRightMotorPin1,LOW);

digitalWrite(BackRightMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 4)

{

digitalWrite(BackLeftMotorPin1,LOW);

digitalWrite(BackLeftMotorPin2,LOW);

}

ledcWrite(PWMSpeedChannel_1, abs(motorSpeed));

ledcWrite(PWMSpeedChannel_2, abs(motorSpeed));

ledcWrite(PWMSpeedChannel_3, abs(motorSpeed));

ledcWrite(PWMSpeedChannel_4, abs(motorSpeed));

}

void setUpPinModes()

{

pinMode(enableFrontRightMotor,OUTPUT);

pinMode(FrontRightMotorPin1,OUTPUT);

pinMode(FrontRightMotorPin2,OUTPUT);

pinMode(enableFrontLeftMotor,OUTPUT);

pinMode(FrontLeftMotorPin1,OUTPUT);

pinMode(FrontLeftMotorPin2,OUTPUT);

pinMode(enableBackRightMotor,OUTPUT);

pinMode(BackRightMotorPin1,OUTPUT);

pinMode(BackRightMotorPin2,OUTPUT);

pinMode(enableBackLeftMotor,OUTPUT);

pinMode(BackLeftMotorPin1,OUTPUT);

pinMode(BackLeftMotorPin2,OUTPUT);

//Set up PWM for motor speed

ledcSetup(PWMSpeedChannel_1, PWMFreq, PWMResolution);

ledcSetup(PWMSpeedChannel_2, PWMFreq, PWMResolution);

ledcSetup(PWMSpeedChannel_3, PWMFreq, PWMResolution);

ledcSetup(PWMSpeedChannel_4, PWMFreq, PWMResolution);

ledcAttachPin(enableBackRightMotor, PWMSpeedChannel_1);

ledcAttachPin(enableBackLeftMotor, PWMSpeedChannel_2);

ledcAttachPin(enableFrontRightMotor, PWMSpeedChannel_3);

ledcAttachPin(enableFrontLeftMotor, PWMSpeedChannel_4);

rotateMotor(BACK_RIGHT_MOTOR, 0);

rotateMotor(BACK_LEFT_MOTOR, 0);

rotateMotor(FRONT_RIGHT_MOTOR, 0);

rotateMotor(FRONT_LEFT_MOTOR, 0);

}

void setup()

{

//

PS4.begin();

uint8_t pairedDeviceBtAddr[20][6];

int count = esp_bt_gap_get_bond_device_num();

esp_bt_gap_get_bond_device_list(&count, pairedDeviceBtAddr);

for(int i = 0; i < count; i++)

{

esp_bt_gap_remove_bond_device(pairedDeviceBtAddr[i]);

}

Serial.println("Previous BT device pairing cleared!!!");

delay(1000);

//

setUpPinModes();

Serial.begin(115200);

PS4.attach(notify);

PS4.attachOnConnect(onConnect);

PS4.attachOnDisconnect(onDisConnect);

PS4.begin("08:d1:f9:e9:58:b2");

Serial.println("Initialization Ready!!!");

}

void loop()

{

}

Stage 2

Control the mecanum wheel robot car wirelessly by another ESP32 with an IMU connected. By tilting and rotating the IMU on your hand, the robot car should be able to move in the directions that is done in Stage 1.

ESP32+IMU+battery

Sender Code:

#include <esp_now.h>

#include <WiFi.h>

#include <Wire.h>

const int IMU_ADDR = 0x68;  // I2C address for IMU. ADO=5V -> 0x69

int16_t accel_X, accel_Y, accel_Z, temp_raw, gyro_X, gyro_Y, gyro_Z;  // variables for raw sensor data

double aX, aY, aZ, theta_acc, phi_acc;                                //accel values and theta and phi calculated from accelerometer

double theta_acc_previous = 0, theta_acc_LP, phi_acc_previous = 0, phi_acc_LP;

double gX, gY, gZ, theta_gyro = 0, phi_gyro = 0, psi_gyro;

double theta_compli, phi_compli;  //theta and phi calculated from accelerometer and gyroscope

unsigned long previousTime, elapseTime;

double dt;

int n_forward, n_backward, n_left, n_right;

//insert mac address of receiver

//uint8_t broadcastAddress[] = {0xE8, 0x68, 0xE7, 0x22, 0xC3, 0xF0};

uint8_t broadcastAddress[] = { 0x3C, 0x61, 0x05, 0x03, 0x95, 0x58 };

// 3C:61:05:03:95:58

//structure of the message to send

//must be matched to the receiver structure

typedef struct struct_message {

int forward;

int backward;

int left;

int right;

} struct_message;

//create a new variable to store the value of "struct_message"

struct_message myData;

//create a new variable to store information about the peer

esp_now_peer_info_t peerInfo;

long serial_print_pmillis = 0;

bool can_print = 0;

//define a callback function that will be executed when a message is sent

//simpfly print if the message was successfully delivered or not

void OnDataSent(const uint8_t *mac_addr, esp_now_send_status_t status) {

//if (can_print) {

Serial.println(WiFi.macAddress());

Serial.print("\r\nLast Packet Send Status:\t");

Serial.println(status == ESP_NOW_SEND_SUCCESS ? "Delivery Success" : "Delivery Failed");

//}

}

void setup() {

// put your setup code here, to run once:

Serial.begin(115200);

//set the device as a WiFi station

WiFi.mode(WIFI_STA);

//initialize ESP-NOW

if (esp_now_init() != ESP_OK) {

Serial.println("Error initializing ESP-NOW");

return;

}

//once ESP-NOW is successfully init

//register a callback function that will be called when a message is sent

esp_now_register_send_cb(OnDataSent);

//pair with anoother ESP-NOW device to send data

//register peer

memcpy(peerInfo.peer_addr, broadcastAddress, 6);

peerInfo.channel = 0;

peerInfo.encrypt = false;

//add peer

if (esp_now_add_peer(&peerInfo) != ESP_OK) {

Serial.println("Failed to add peer");

return;

}

//setup for I2C communicate of mpu9250

Wire.begin();

Wire.beginTransmission(IMU_ADDR);  //communicate with IMU

Wire.write(0x6B);                  //PWR_MGMT_1 register

Wire.write(0x0);

Wire.endTransmission(true);

Wire.beginTransmission(IMU_ADDR);  //communicate with IMU

Wire.write(0x1C);                  //ACCEL_CONFIG register

Wire.write(0b00000000);            //Bit3 & Bit 4 -> 0,0 -> +/-2g

Wire.endTransmission(true);

Wire.beginTransmission(IMU_ADDR);  //communicate with IMU

Wire.write(0x1B);                  //GYRO_CONFIG register

Wire.write(0b00000000);            //Bit3 & Bit 4 -> 0,0 -> +/-250deg/s

Wire.endTransmission(true);

Wire.beginTransmission(IMU_ADDR);  //communicate with IMU

Wire.write(0x37);                  //  IMU INT PIN CONFIG

Wire.write(0x02);                  //  0x02 activate bypass in order to communicate with magnetometer

Wire.endTransmission(true);

delay(100);

previousTime = millis();

}

void loop() {

can_print = 0;//millis() - serial_print_pmillis > 500;

if (millis() - serial_print_pmillis > 500) serial_print_pmillis = millis();

// put your main code here, to run repeatedly:

Wire.beginTransmission(IMU_ADDR);

Wire.write(0x3B);  // start from ACCEL_XOUT_H

Wire.endTransmission(false);

Wire.requestFrom(IMU_ADDR, 14, true);  //request a total of 14 bytes

// read registers from IMU

accel_X = Wire.read() << 8 | Wire.read();   //read registers 0x3B (ACCEL_XOUT_H) & 0x3C (ACCEL_XOUT_L)

accel_Y = Wire.read() << 8 | Wire.read();   //read registers 0x3D & 0x3E

accel_Z = Wire.read() << 8 | Wire.read();   //read registers 0x3F & 0x40

temp_raw = Wire.read() << 8 | Wire.read();  //read registers 0x41 & 0x42

gyro_X = Wire.read() << 8 | Wire.read();    //read registers 0x43  (GYRO_XOUT_H) & 0x44 (GYRO_XOUT_L)

gyro_Y = Wire.read() << 8 | Wire.read();    //read registers 0x45 & 0x46

gyro_Z = Wire.read() << 8 | Wire.read();    //read registers 0x47 & 0x48

Wire.endTransmission(true);

aX = accel_X / 16384.0;  // convert the raw data to g value. 1g = 9.8 m/s2

aY = accel_Y / 16384.0;

aZ = accel_Z / 16384.0;

if (can_print) {

Serial.print("ax:\t");

Serial.print(aX);

Serial.print(",ay:\t");

Serial.print(aY);

Serial.print(",az:\t");

Serial.println(aZ);

}

//--------------Getting Roll(phi) and Pitch(theta) from accelerometer readings only-------------------//

theta_acc = atan2(aX, aZ) * (180 / PI) - 44;  // approximation of theta (pitch). use either this or the eauation right below.

// theta_acc = atan2(aX, sqrt(pow(aY,2)+pow(aZ,2)))*(180/PI); // another approximation on theta (pitch). Either use this or the equation right above.

phi_acc = atan2(aY, aZ) * (180 / PI) + 44;  // approximation of phi (roll). use either this or the eauation right below.

// phi_acc = atan2(aY, sqrt(pow(aX,2)+pow(aZ,2)))*(180/PI);// another approximation on phi (roll). Either use this or the equation right above.

// simple Low Pass Filter (LPF) added for theta(pitch) and phi(roll)

theta_acc_LP = 0.7 * theta_acc_previous + 0.3 * theta_acc;  // adjust the ratio of preivous theta value and the newly measured value

phi_acc_LP = 0.7 * phi_acc_previous + 0.3 * phi_acc;        //gs only-----

theta_acc_previous = theta_acc_LP;

phi_acc_previous = phi_acc_LP;

//------------end of Roll(phi) and Pitch(theta) from accelerometer readings---------//

//--------------Getting Roll(phi) and Pitch(theta) from gyroscope readin--------------//

gX = gyro_X / 131.0 - 5.8;  // convert the raw data to rad/s. "- 5.8" is the gyro error

gY = gyro_Y / 131.0 - 6;

gZ = gyro_Z / 131.0;

elapseTime = millis() - previousTime;

dt = elapseTime / 1000.0;

previousTime = millis();

theta_gyro = theta_gyro - gY * dt;  // pitch angle from gyro

phi_gyro = phi_gyro + gX * dt;      // roll angle from gyro

psi_gyro = psi_gyro + gZ * dt;      // yaw angle from gyro (not quite useful, only incremental value)

//-----------end of Roll(phi) and Pitch(theta) from gyroscope readings --------------------//

// ------- Complimentary filter from accelerometer and gyroscope data ---------//

// Complimentary filter uses a mix of two sensor data with

//a specific ratio which depend on which one you trust better

theta_compli = 0.95 * (theta_compli - gY * dt) + 0.05 * (theta_acc);

phi_compli = 0.95 * (phi_compli + gX * dt) + 0.05 * (phi_acc);

// ------- end of Complimentary filter from accelerometer and gyroscope data -----//

//-------conditional limition of reading value (lowerLimit - larger than && upperLimit - smaller than)-------//

if (theta_acc < -8 && theta_acc > -50 && phi_acc < -2 && phi_acc > -40) { n_forward = 1; }  //statement of forward

else {

n_forward = 0;

}

if (theta_acc > 10 && theta_acc < 50 && phi_acc < -2 && phi_acc > -40) { n_backward = 1; }  //statement of backward

else {

n_backward = 0;

}

if (theta_acc > 2 && theta_acc < 60 && phi_acc < -12 && phi_acc > -50) { n_left = 1; }  //statement of left

else {

n_left = 0;

}

if (theta_acc > 3 && theta_acc < 50 && phi_acc > 10 && phi_acc < 120) { n_right = 1; }  //statement of right

else {

n_right = 0;

}

//------------------------------End of limition-------------------------------//

//set the variables values to send

if (can_print) Serial.print("255*aY:\t");

if (can_print) Serial.println(255 * aY);

myData.forward = abs(255 * aY * n_forward);

myData.backward = 255 * aY * !(n_forward || n_left || n_right);

myData.left = constrain(map(aX, 1, -1, 255, -255),-255,255);

myData.right = constrain(map(aX, 1, -1, -255, 255),-255,255);

/*

//set the variables values to send

myData.forward = n_forward;

myData.backward = n_backward;

myData.left = n_left;

myData.right = n_right;

*/

if (can_print) Serial.print("theta_acc :");

if (can_print) Serial.println(theta_acc);

if (can_print) Serial.print("phi_acc :");

if (can_print) Serial.println(phi_acc);

//  Serial.print("theta_acc_LP :"); Serial.println(theta_acc_LP);

//  Serial.print("phi_acc_LP :"); Serial.println(phi_acc_LP);

//  Serial.print("theta_gyro :"); Serial.println(theta_gyro);

//  Serial.print("phi_gyro :"); Serial.println(phi_gyro);

//  Serial.print("psi_gyro :"); Serial.println(psi_gyro);

//  Serial.print("theta_compli :"); Serial.println(theta_compli);

//  Serial.print("phi_compli :"); Serial.println(phi_compli);

if (can_print) {

Serial.print("forward :");

Serial.println(n_forward);

Serial.print("backward :");

Serial.println(n_backward);

Serial.print("left :");

Serial.println(n_left);

Serial.print("right :");

Serial.println(n_right);

}

//send the message via ESP-NOW

esp_err_t result = esp_now_send(broadcastAddress, (uint8_t *)&myData, sizeof(myData));

if (result == ESP_OK) {

/*if (can_print)*/ Serial.println("Sent with success");

} else {

/*if (can_print)*/ Serial.println("Error sending the data");

}

//delay(50);

}

Code Receiver:

#include <esp_now.h>

#include <WiFi.h>

#define FORWARD 1

#define BACKWARD 2

#define LEFT 3

#define RIGHT 4

#define FORWARD_LEFT 5

#define FORWARD_RIGHT 6

#define BACKWARD_LEFT 7

#define BACKWARD_RIGHT 8

#define ROTATE_LEFT 9

#define ROTATE_RIGHT 10

#define STOP 0

#define BACK_RIGHT_MOTOR 1

#define BACK_LEFT_MOTOR 2

#define FRONT_RIGHT_MOTOR 3

#define FRONT_LEFT_MOTOR 4

#define speed 130 // about 50% duty cycle

#define MAX_MOTOR_SPEED 255

unsigned long lastTimeStamp = 0;

//FRONT RIGHT MOTOR

int enableFrontRightMotor = 14;

int FrontRightMotorPin1 = 26;

int FrontRightMotorPin2 = 27;

//BACK RIGHT MOTOR

int enableBackRightMotor=22;

int BackRightMotorPin1=16;

int BackRightMotorPin2=17;

//FRONT LEFT MOTOR

int enableFrontLeftMotor = 32;

int FrontLeftMotorPin1 = 33;

int FrontLeftMotorPin2 = 25;

//BACK LEFT MOTOR

int enableBackLeftMotor = 23;

int BackLeftMotorPin1 = 18;

int BackLeftMotorPin2 = 19;

const int PWMFreq = 1000; /* 1 KHz */

const int PWMResolution = 8; // 8-bit

const int PWMSpeedChannel_1 = 1; // 0-15

const int PWMSpeedChannel_2 = 2; // 0-15

const int PWMSpeedChannel_3 = 3; // 0-15

const int PWMSpeedChannel_4 = 4; // 0-15

void moveCar(int inputValue)

{

switch(inputValue)

{

case FORWARD:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case BACKWARD:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case FORWARD_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case FORWARD_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

case BACKWARD_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

case BACKWARD_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case ROTATE_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case ROTATE_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case STOP:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

default:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

}

}

void moveCar(int inputValue,byte Speed)

{

switch(inputValue)

{

case FORWARD:

rotateMotor(FRONT_RIGHT_MOTOR, Speed);

rotateMotor(BACK_RIGHT_MOTOR, Speed);

rotateMotor(FRONT_LEFT_MOTOR, Speed);

rotateMotor(BACK_LEFT_MOTOR, Speed);

break;

case BACKWARD:

rotateMotor(FRONT_RIGHT_MOTOR, -Speed);

rotateMotor(BACK_RIGHT_MOTOR, -Speed);

rotateMotor(FRONT_LEFT_MOTOR, -Speed);

rotateMotor(BACK_LEFT_MOTOR, -Speed);

break;

case LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, Speed);

rotateMotor(BACK_RIGHT_MOTOR, -Speed);

rotateMotor(FRONT_LEFT_MOTOR, -Speed);

rotateMotor(BACK_LEFT_MOTOR, Speed);

break;

case RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -Speed);

rotateMotor(BACK_RIGHT_MOTOR, Speed);

rotateMotor(FRONT_LEFT_MOTOR, Speed);

rotateMotor(BACK_LEFT_MOTOR, -Speed);

break;

case FORWARD_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, Speed);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, Speed);

break;

case FORWARD_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, Speed);

rotateMotor(FRONT_LEFT_MOTOR, Speed);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

case BACKWARD_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, -Speed);

rotateMotor(FRONT_LEFT_MOTOR, -Speed);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

case BACKWARD_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -Speed);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, -Speed);

break;

case ROTATE_LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, Speed);

rotateMotor(BACK_RIGHT_MOTOR, Speed);

rotateMotor(FRONT_LEFT_MOTOR, -Speed);

rotateMotor(BACK_LEFT_MOTOR, -Speed);

break;

case ROTATE_RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -Speed);

rotateMotor(BACK_RIGHT_MOTOR, -Speed);

rotateMotor(FRONT_LEFT_MOTOR, Speed);

rotateMotor(BACK_LEFT_MOTOR, Speed);

break;

case STOP:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

default:

rotateMotor(FRONT_RIGHT_MOTOR, STOP);

rotateMotor(BACK_RIGHT_MOTOR, STOP);

rotateMotor(FRONT_LEFT_MOTOR, STOP);

rotateMotor(BACK_LEFT_MOTOR, STOP);

break;

}

}

void rotateMotor(int motorNumber, int motorSpeed)

{

if (motorSpeed < 0 && motorNumber == 1)

{

digitalWrite(BackRightMotorPin1,LOW);

digitalWrite(BackRightMotorPin2,HIGH);

}

else if (motorSpeed < 0 && motorNumber == 2)

{

digitalWrite(BackLeftMotorPin1,LOW);

digitalWrite(BackLeftMotorPin2,HIGH);

}

else if (motorSpeed < 0 && motorNumber == 3)

{

digitalWrite(FrontRightMotorPin1,LOW);

digitalWrite(FrontRightMotorPin2,HIGH);

}

else if (motorSpeed < 0 && motorNumber == 4)

{

digitalWrite(FrontLeftMotorPin1,LOW);

digitalWrite(FrontLeftMotorPin2,HIGH);

}

else if (motorSpeed > 0 && motorNumber == 1)

{

digitalWrite(BackRightMotorPin1,HIGH);

digitalWrite(BackRightMotorPin2,LOW);

}

else if (motorSpeed > 0 && motorNumber == 2)

{

digitalWrite(BackLeftMotorPin1,HIGH);

digitalWrite(BackLeftMotorPin2,LOW);

}

else if (motorSpeed > 0 && motorNumber == 3)

{

digitalWrite(FrontRightMotorPin1,HIGH);

digitalWrite(FrontRightMotorPin2,LOW);

}

else if (motorSpeed > 0 && motorNumber == 4)

{

digitalWrite(FrontLeftMotorPin1,HIGH);

digitalWrite(FrontLeftMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 1)

{

digitalWrite(FrontRightMotorPin1,LOW);

digitalWrite(FrontRightMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 2)

{

digitalWrite(FrontLeftMotorPin1,LOW);

digitalWrite(FrontLeftMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 3)

{

digitalWrite(BackRightMotorPin1,LOW);

digitalWrite(BackRightMotorPin2,LOW);

}

else if (motorSpeed == 0 && motorNumber == 4)

{

digitalWrite(BackLeftMotorPin1,LOW);

digitalWrite(BackLeftMotorPin2,LOW);

}

ledcWrite(PWMSpeedChannel_1, abs(motorSpeed));

ledcWrite(PWMSpeedChannel_2, abs(motorSpeed));

ledcWrite(PWMSpeedChannel_3, abs(motorSpeed));

ledcWrite(PWMSpeedChannel_4, abs(motorSpeed));

}

void setUpPinModes()

{

pinMode(enableFrontRightMotor,OUTPUT);

pinMode(FrontRightMotorPin1,OUTPUT);

pinMode(FrontRightMotorPin2,OUTPUT);

pinMode(enableFrontLeftMotor,OUTPUT);

pinMode(FrontLeftMotorPin1,OUTPUT);

pinMode(FrontLeftMotorPin2,OUTPUT);

pinMode(enableBackRightMotor,OUTPUT);

pinMode(BackRightMotorPin1,OUTPUT);

pinMode(BackRightMotorPin2,OUTPUT);

pinMode(enableBackLeftMotor,OUTPUT);

pinMode(BackLeftMotorPin1,OUTPUT);

pinMode(BackLeftMotorPin2,OUTPUT);

//Set up PWM for motor speed

ledcSetup(PWMSpeedChannel_1, PWMFreq, PWMResolution);

ledcSetup(PWMSpeedChannel_2, PWMFreq, PWMResolution);

ledcSetup(PWMSpeedChannel_3, PWMFreq, PWMResolution);

ledcSetup(PWMSpeedChannel_4, PWMFreq, PWMResolution);

ledcAttachPin(enableBackRightMotor, PWMSpeedChannel_1);

ledcAttachPin(enableBackLeftMotor, PWMSpeedChannel_2);

ledcAttachPin(enableFrontRightMotor, PWMSpeedChannel_3);

ledcAttachPin(enableFrontLeftMotor, PWMSpeedChannel_4);

rotateMotor(BACK_RIGHT_MOTOR, 0);

rotateMotor(BACK_LEFT_MOTOR, 0);

rotateMotor(FRONT_RIGHT_MOTOR, 0);

rotateMotor(FRONT_LEFT_MOTOR, 0);

}

// structure example to receive data

// must be matched to the sender structure

typedef struct struct_message {

int forward;

int backward;

int left;

int right;

} struct_message;

// Create a struct_message called myData

struct_message myData;

// callback function that will be executed when data is received

void OnDataRecv(const uint8_t * mac, const uint8_t *incomingData, int len) {

memcpy(&myData, incomingData, sizeof(myData));

Serial.print("Bytes received: ");

Serial.println(len);

Serial.print("forward: ");

Serial.println(myData.forward);

Serial.print("backward: ");

Serial.println(myData.backward);

Serial.print("left: ");

Serial.println(myData.left);

Serial.print("right: ");

Serial.println(myData.right);

Serial.println();

}

void setup() {

// put your setup code here, to run once:

// Initialize Serial Monitor

Serial.begin(115200);

// Set device as a Wi-Fi Station

WiFi.mode(WIFI_MODE_STA);

// Init ESP-NOW

if (esp_now_init() != ESP_OK) {

Serial.println("Error initializing ESP-NOW");

return;

}

// Once ESPNow is successfully Init, we will register for recv CB to

// get recv packer info

esp_now_register_recv_cb(OnDataRecv);

setUpPinModes();

}

void loop() {

// put your main code here, to run repeatedly:

if (myData.forward  >= 10) {moveCar(FORWARD ,myData.forward );}

else if (myData.backward >= 10) {moveCar(BACKWARD,myData.backward);}

else if (myData.left     >= 10) {moveCar(LEFT    ,myData.left    );}

else if (myData.right    >= 10) {moveCar(RIGHT   ,myData.right   );}

/*

if (myData.forward == 1) {moveCar(FORWARD);}

else if (myData.backward == 1) {moveCar(BACKWARD);}

else if (myData.left == 1) {moveCar(LEFT);}

else if (myData.right == 1) {moveCar(RIGHT);}

*/

//  else if (RStickX < -50) {moveCar(ROTATE_LEFT);}

//  else if (RStickX > 50) {moveCar(ROTATE_RIGHT);}

//  else if (LStickY > 50 && LStickX < -50) {moveCar(FORWARD_LEFT);}

//  else if (LStickY > 50 && LStickX > 50) {moveCar(FORWARD_RIGHT);}

//  else if (LStickY < -50 && LStickX < -50) {moveCar(BACKWARD_LEFT);}

//  else if (LStickY < -50 && LStickX > 50) {moveCar(BACKWARD_RIGHT);}

else{moveCar(STOP);}

}

Example code

case FORWARD:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case BACKWARD:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

case LEFT:

rotateMotor(FRONT_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, MAX_MOTOR_SPEED);

break;

case RIGHT:

rotateMotor(FRONT_RIGHT_MOTOR, -MAX_MOTOR_SPEED);

rotateMotor(BACK_RIGHT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(FRONT_LEFT_MOTOR, MAX_MOTOR_SPEED);

rotateMotor(BACK_LEFT_MOTOR, -MAX_MOTOR_SPEED);

break;

Reference:

https://github.com/wysh2503/MBS4544_Robot_Sensing_and_Vision/blob/main/PS4-ESP32-MecanumWheel/PS4-ESP32-MecanumWheel.ino